Midterm Mega Quiz File

DMA stands for_________

►Direct Memory Access (Page 4)

►Distinct Memory Access

►Direct Module Access

►Direct Memory Allocation

Following is not a method of I/O

►Programmed I/O

► Interrupt driven I/O

►Hardware Based I/O (Page 4)

►None of given

The Function of I/O controller is to provide ____________.

►I/O control signals

►Buffering

►Error Correction and Detection

►All of given (Page 5)

Standard PC operates in two modes in terms of memory which are

►Real mode and Extended Mode

►Base mode and Memory Mode

►None of the given

►Real mode and protected mode (Page 6)

There are two main types of interrupts namely _________________.

►PC based and Window based

►Hardware based and Kernal based

►Hardware interrupts and Software interrupts (Page 10)

►None of the given

Interrupt Vector Table (IVT) in short is a _______ bytes sized table.

►1024 (Page 10)

►2048

►3072

►4096

Maximum number of interrupts in a standard PC is

► 64

► 128

► 256 (Page 10)

► 512

Which of the following are types of ISR __________.

►BIOS (Basic I/O service ) ISR

►DOS ISR

►ISR provided by third party device drivers

►All of the given (Page 13)

IVT is a table containing ______ byte entries each of which is a far address of an interrupt service routine.

►2

►4 (Page 20)

►8

►16

To set the interrupt vector means is to change the double word sized interrupt vector within the IVT.

►True (Page 22)

►False

Set the Interrupt vector means to change the double word sized interrupt vector within IVT.

►True (Page 22)

►False

Each paragraph in keep function is ____ bytes in size.

►4

►8

►16 (Page 24)

►32

Interrupt service number is usually placed in the ________ register.

►AL

►CL

►AH (Page 26)

►AX

Timer interrupt is a _________.

►Hardware Interrupt (Page 28)

►Software Interrupt

►Both of these

►None of these

Timer interrupt occurs _______ times every second by means of hardware.

►18.2 (Page 28)

►16.2

►15.2

►14.2

Keyboard Status Byte is located at the address

► 0040:0000H

► 0040:0013H

► 0040:0015H

► 0040:0017H (Page 29)

The output on the monitor is controlled by a controller called __________within the PC.

►Video controller (Page 30)

►Bus controller

►Ram controller

►None of the given

Display device (Monitor) performs _________ I/O.

►memory mapped (Page 30)

►Isolated

►Both of above

►None of these

The keyboard makes use of interrupt number _______ for its input operations.

►9 (Page 34)

►10

►11

►12

Interrupt 9 usually reads the _________ from keyboard.

►ASCII code

►Scan code (Page 34)

►Both ASCII and Scan code

►None of the above

Usually interrupt procedures are reentrant procedures especially those interrupt procedure compiled using C language compiler are reentrant.

►True (Page 38)

►False

______ is Disk interrupt.

►10H

►11H

►13H (Page 42)

►14H

The service _________ is called the keyboard hook service.

►15H/2FH

►15H/4FH (Page 44)

►15H/FFH

NMI Stand for

►Non Maskable Interrupt (Page 46)

►Non Multitude Interrupt

►Non Maskable Instruction

►None of Given

The microprocessor package has many signals for data. Below are some in Correct priority order (Higher to Lower).

►Reset,Hold,NMI,INTR (Page 46)

►NMI, INTR,Hold,Reset

►INTR,NMI,Reset,Hold

►None of the Given

A single interrupt controller can arbitrate among ____ different devices.

►4

►6

►8 (Page 47)

►10

Hardware Interrupts are __________.

►Preemptive

►Non-Preemptive (Page 48)

►Both Preemptive and Non-Preemptive

►None of given

A software interrupt does not require EOI (End of interrupt).

►True (Page 49)

►False

In keyboard status byte bit no. 2 and 3 are used for ctrl and alt keys respectively. which of the following condition is used to check that Ctrl + Alt keys are pressed. Where: unsigned char far * scr = (unsigned char far *)(0x00400017);

►if (((*scr)&12)==12) (Page 52)

►if (((*scr)&8)==8)

►if (((*scr)&4)==4)

►if (((*scr)&2)==2)

To store each character in keyboard buffer ______________bytes are required

►2 (Page 54)

►4

►6

►8

____ No. of bytes are used to store the character in the keyboard buffer.

► 1

►2 (Page 54)

►4

► 8

Total No. of bytes that can be stored in Keyboard Buffer is____.

►16

►32 (Page 54)

►64

►128

If keyboard buffer is empty the head and tail points at the same location.

►True (Page 55)

►False

Tail of keyboard should get to get the start of buffer.

►True (Page 55)

►False

Interrupt _____ is empty; we can use its vector as a flag.

►9H

►13H

►15H

►65H (Page 65)

Register can be used to divide frequency is _________

►Counter Register (Page 69)

►Accumulator Register

►None of these

Counter register can be used to divide clock signal.

►True (Page 69)

►False

In counter register bit no. 3 changes its value between 0 and 1 with in ____clock cycles

►1

►2

►4

► 16 (Page 69)

Command register is an _____ bit register

►4

►8 (Page 71)

►16

►32

The interval timer can operate in _______ modes.

►Five

►Seven

►Four

►Six (Page 72)

The following command “outportb (0x61,inportb(0x61) & 0xFC);” will

►Turn on the speaker (Page 74)

►Turn off the speaker

►Toggle the speaker

►None of the given

PPI stands for

►Parallel Programmable interface

►Peripheral Programmable interface (Page 76)

►Port Programmable interface

►None of the given

PPI is used to perform parallel communication

►True (Page 81)

►False

The PPI acts as an interface between the CPU and a parallel ________ .

►I/O device (Page 83)

►CPU

►BUS

►None of given

An I/O device cannot be directly connected to the busses so controller is placed between CPU and I/O.

►True (Page 83)

►False

__________is used to control the printer via the BIOS

►Int 16H

►Int 17H (Page 84)

►Int 18H

►Int 19H

Standard PC can have _____ PPI.

►1

►4 (Page 84)

►8

►16

BIOS DO NOT support ______.

►LPT1

►LPT2

►LPT3

►LPT4 (Page 91)

LPTs can be swapped.

►True (Page 92)

►False

________ store the base address for LPT1.

►40:00H

►40:02H

►40:08H (Page 92)

►40:1AH

At IRQ 7 Interrupt # ___ is used.

► 0x0A

►0x0B

► 0x0C

►0x0F (Page 95)

If printer is _____ then printer sends back the ACK signal to the printer interface

►idle (Page 97)

►busy

►Out of paper

►None of the given

PPI interconnection _______ bits is cleared to indicate low nibble is being sent.

►D1

►D2

► D3

► D4 (Page 101)

_____ bit is cleared to indicate the low nibble is being sent.

►D1

►D2

►D3

►D4 (Page 104)

There are __________ kinds of serial communication.

►2 (Page 105)

►3

►4

►5

In case of synchronous communication a timing signal is required to identify the start and end of a bit.

►True (Page 105)

►False

In ____________each byte is needed to be encapsulated in start and end.

►Synchronous communication

►Asynchronous communication (Page 106)

►Both

►None of given

UART stands for_______

►Universal Asynchronous Receiver Transmitter (Page 107)

►Universal Adjustment and Realigning Tool

►Unconventional Assisted Recovery Team

►None of these

DCE stands for __________.

►Data communication equipment (Page 109)

►Distributed Computing Environment

►Data Communications Equipment

►Data Carrier Equipment

DTE is ____________.

►Data terminal equipment (Page 109)

►Data transmitting equipment

►Dual terminal equipment

►None of the given.

DSR stands for __________ .

►Data set ready (Page 111)

►Data service ready

►Data stock ready

►None of the given

BIOS supports _____________UARTS as COM ports.

►6

►4 (Page 113)

►3

►2

The bit ______ of Line control register in UART, if cleared will indicate that DLL is the data register.

►1

►3

►5

►7 (Page 114)

The baud rate is set in accordance with the divisor value loaded within the UART internal registers base +0 and base +1.

►TRUE (Page 114)

►FALSE

_________ is used to identify the cause of interrupt.

►Interrupt ID Register (Page 116)

►PC Register

►AC Register

►None of All These

In self test mode the output of the UART is routed to its input.

►True (Page 117)

►False

Int 14H __________ can be used to set the line parameter of the UART or COM port.

►Service # 0 (Page 119)

► Service # 1

►Service # 2

►None of the given options

Int 14H____________ can be used to send a byte

►Service#0

►Service#1 (Page 121)

►Service#2

►None of the given option

Int 14H __________ can be used to receive a byte.

►Service # 0

►Service # 1

► Service # 2 (Page 121)

►None of the given options

The -------- function receive a byte and COM port number is passed as parameter using BIOS service

►Receivebyte ();

►Receive ();

►Receivechar (); (Page 125)

►None of the given option

The________ function uses the COM port number to receive a byte from the COM port using BIOS services.

►recievebyte()

►initialize ()

►receive()

►recievechar() (Page 125)

The ________function initialize the COM port whose number is passed as parameter using BIOS services.

►Initializecom()

►Initialize() (Page 125)

►Recievechar()

►None of these option

The ____________ function simply enables the self test facility within the modem control register

►STOn()

►SelfTest()

►SelfTestOn() (Page 127)

►None of these

____whenever received indicates the start of communication and ______ whenever received indicates a temporary pause in the communication.

►XON & XOFF (Page 135)

►XOFF & XON

►XON & YOFF

►YON & XOFF

------------ whenever receive indicates the start of communication ........... whenever receive indicates the end of communication

►XON\XOFF (Page 135)

►XOFF\XON

►XON\YOFF

►YON\XOFF

Software based flow control make use of -------- control characters

►Xon

►XOFF

►Both (Page 135)

►None

The BIOS interrupt 0x1AH can be used to configure real time clock

►True (Page 136)

►False

________ is a device incorporated into the PC to update time even if the computer is off.

►Clock counter

►ROM

►Clock

►Real time clock (Page 136)

The BIOS interrupt ________ can be used to configure RTC.

►1AH (Page 136)

►2AH

►3AH

►4AH

Interrupt ______ is used to get or set the time.

►0AH

►1AH (Page 136)

►2AH

►3AH

------------ is used to read time from RTC

►1A\02H (Page 137)

►1A\03H

►1A\04H

►1A\05H

------------ is used to set time from RTC

►1A\02H

►1A\03H (Page 138)

►1A\04H

►1A\05H

------------ is used to read date from RTC

►1A\02H

►1A\03H

►1A\04H (Page 138)

►1A\05H

Only ________ ports are important from programming point of view.

► 70 and 71H (Page 141)

►71 and 72H

►70 and 72H

►72 and 73H

The amount of memory above conventional memory (extended memory) can be determined using the service _______.

►15H/88H (Page 162)

►16H/88H

►17H/88H

►21H/88H

_________ used to determine the amount of conventional memory interfaced with the processor in kilobytes.

►INT 10 H

►INT 11 H

►INT 12 H (Page 162)

►INT 13 H

To distinguish 486 with Pentium CPUID Test is used.

►True (Page 166)

►False

If CPUID instruction is not present then the processor can be a

► 486 processor (Page 166)

► 386 processor

► 286 processor

► All of the above

Bit number _______ of coprocessor control word is the Interrupt Enable Flag.

►7 (Page 168)

►8

►9

►10

The bit number _______ of the coprocessor control word is the interrupt enable flag.

►7 (Page 168)

►8

►9

►6

Keyboard uses port ____ as status port.

►64H (Page 177)

►66H

►67H

►69H

Which port is known as Data Port______

►60H (Page 177)

►61H

►64H

►69H

The keyboard input character scan code is received at ___ port.

►60H (Page 179)

►61H

►62H

►63H

________means typematic rate will be sent in next byte.

►0xF3 (Page 180)

►0xF4

►0xF5

►0xF6

_______ is LED control byte.

►0xFD

►0xED (Page 181)

►0xFF

►0xEE

By cascading two DMAs ____ bits can be transferred.

►4

►8

►16 (Page 186)

►32